## Delmhorst Instrument Co.

## CATALOG/WEBSITE COPY FOR

TM-100

The Model TM-100 measures temperature over the range of -40ºF to 300ºF, -40ºC to 150ºC. Ideal for various applications in industry and agriculture, this instrument incorporates reliable semiconductor technology in a sturdy ABS plastic case. The RTD sensor is housed in stainless steel to withstand the most demanding applications.

### TM-100 Meter

Features:

- 3 1/2" Digital LCD readout

- Temperature stable circuit

- Low battery indicator

- Membrane switches with positive tactile feel

- Carrying case included

- 1 year limited warranty

## Delmhorst Instrument Co.

## CATALOG/WEBSITE COPY FOR

TM-100

Specifications:

- Readout Mode: ºF, ºC (selectable)

- Moisture Range:

  - -40ºF to 300ºF

  - -40ºC to 150ºC

- Accuracy:

  - 40ºC to 80ºC, ±1.5ºC

  - 80ºC to 150ºC, ±2.5ºC

  - -40ºF to 176ºF, ±3ºF

  - 176ºF to 300ºF, ±4.5ºF

- Resolution:

  - 0.1ºF from -40ºF to 199.9ºF

  - 1ºF from 200ºF to 300ºF

  - 0.1ºC to 150ºC

- Response Time:

  - 2 minutes for 63% excursion

  - 4-5 minutes full excursion

(NOTE: Response time for bare sensor is 30 seconds. The heavy duty housing required for some applications is the reason for a slower overall response time.)