August 2013

| Americas Bosch Security Systems, Inc. 130 Perinton Parkway Fairport, New York, 14450, USA Phone: + 1 800 289 0096 Fax: +1 585 223 9180 security.sales@us.bosch.com www.boschsecurity.us | Europe, Middle East, Africa Bosch Security Systems B.V. P.O. Box 80002 5600 JB Eindhoven, The Netherlands Phone: + 31 40 2577 284 Fax: +31 40 2577 330 emea.securitysystems@bosch.com www.boschsecurity.com | Asia-Pacific Robert Bosch (SEA) Pte Ltd, Security Systems 11 Bishan Street 21 Singapore 573943 Phone: +65 6571 2600 Fax: +65 6571 2698 apr.securitysystems@bosch.com www.boschsecurity.com |
| --- | --- | --- |

Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2004 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

SECTION 28 23 19

DIGITAL VIDEO RECORDERS AND ANALOG RECORDING DEVICES

BOSCH DSA E-SERIES iSCSI DISK ARRAYS

- – GENERAL

  - SUMMARY

    - Section Includes

      - Security Lighting

    - Related Sections

      - Section [28 23 13 – Video Surveillance Control and Management Systems]

      - Section [28 23 16 – Video Surveillance Monitoring and Supervisory Interfaces]

      - Section [28 23 23 – Video Surveillance Systems Infrastructure]

      - Section [28 23 29 – Video Surveillance Remote Devices]

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

    - Australia and New Zealand Standards

      - CISPR 22 (2008)

      - CISPR 24 (1997) + A1, A2

      - CISPR 22: 2005+A1: 2005, CLASS A

    - Emissions/Immunity

      - VCCI V3/2010.4, Class A

    - European Norm (CE)

      - Complies with CE requirements

      - EN 60950-1 Second Edition

      - EN 60950-1:2006/A11:2009/A1:2010/A12:2011

      - EN55022 (2006) + A1(2007) /

      - EN55024 (1998) + A1 (2001) + A2 (2003)

      - EN61000-3-2 (2006) + A1 + A2 (2009)

      - EN61000-3-3 (2008)

      - EN 50581:2012

      - ICES-003: 2004, Class A

    - Federal Communications Commission (FCC) (www.fcc.gov)

      - FCC Part 15, Subpart B, Class A

    - International Electrotechnical Commission (IEC)

      - IEC 60950-1:2005 Second Edition: Am1: 2009

      - IEC 61000-3-2 (2005) Harmonics

      - IEC 6100-3-3 (2008) Flicker

    - International Organization for Standardization (ISO)

      - 9001 – Quality System

    - Underwriters Laboratories, Inc. (UL) (www.ul.com)

      - EN 60950-1:2006/ A11:2009 + A1:2010 + A12:2011

  - DEFINITIONS

    - RAID: Redundant Array of Independent Disks is a technology that employs the simultaneous use of two or more hard disk drives to achieve greater levels of performance, reliability, and/or larger data volume sizes.

    - iSCSI: Internet SCSI is a protocol that allows clients (called initiators) to send SCSI commands to SCSI storage devices (targets) on remote servers via an IP network.

    - Battery-backed Cache: Non-volatile cache memory is a type of random access memory which does not lose its information when power is turned off.

    - SNMP: Simple Network Management Protocol is a component of the Internet Protocol Suite as defined by the Internet Engineering Task Force (IETF). SNMP is used in network management systems to monitor network-attached devices for conditions that warrant administrative attention.

  - SYSTEM DESCRIPTION

    - Section Includes

      - Digital Video Recorders and Analog Recording Devices

    - Performance Requirements

      - The iSCSI Disk Array shall offer a scale-out network storage solution.

      - The iSCSI Disk Array shall offer a controller unit with 12 internal HDDs with capacity to support up to 192 HDDs via SAS interface-connected expansion units.

      - The iSCSI Disk Array shall offer a configurable hardware-based RAID protection (RAID-5 or RAID-6) for highest system performance.

      - The iSCSI Disk Array shall offer redundant hot-swappable power supplies and cooling fans.

      - The iSCSI Disk Array base unit shall provide a maximum of 12 x 2 TB Serial Attached SCSI hard disk drives (Nearline SAS).

      - The iSCSI Disk array shall offer two optional expansion shelves:

        - A 12-drive standard shelf

        - A 60-drive 4U high-density shelf.

      - The iSCSI Disk Array shall offer two 10 Gigabit Ethernet ports for high-speed iSCSI connectivity.

      - The iSCSI Disk Array shall be fully modular for ease of service.

  - SUBMITTALS

    - Submit under provisions of Section [01 33 00.]

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Shop Drawings; include

      - System device locations on architectural floor plans.

      - Full Schematic of system, including wiring information for all devices.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - System device locations on architectural floor plans.

      - Wiring and connection diagram.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - This product shall be manufactured by a firm whose quality system is in compliance with the I.S. /ISO 9001/EN 29001, QUALITY SYSTEM.

    - Installer:

      - Minimum of [5] years experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements of Section 01 60 00.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

    - Bosch provides off-the-shelf availability for our top selling products and same-day or 24-hour shipping.

  - WARRANTY

    - Provide manufacturer’s warranty covering 3 years for replacement and repair of defective equipment.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support from 8:00 a.m. to 8:00 p.m. via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

[Bosch Security Systems, Inc.

130 Perinton Parkway

Fairport, New York, 1450, USA

Phone: + 1 800 289 0096

Fax: + 1 585 223 9180

security.sales@us.bosch.com

www.boschsecurity.us]

[Bosch Security Systems B.V.

P.O. Box 80002

5600 JB Eindhoven, The Netherlands

Phone: + 31 40 2577 284

Fax: +31 40 2577 330

emea.securitysystems@bosch.com

www.boschsecurity.com]

[Robert Bosch (SEA) Pte Ltd, Security Systems

11 Bishan Street 21

Singapore 573943

Phone: +65 6571 2600

Fax: +65 6571 2698

apr.securitysystems@bosch.com

www.boschsecurity.com]

    - Substitutions: [Not permitted.] [Under provisions of Division 1.]

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - BOSCH DSA E SERIES iSCSI DISK ARRAY [DSA-N2C6X2-12AT] 
[DSX-N1D6X2-12AT] [DSX-N6D6X3-60AT]

    - General Characteristics:

      - The iSCSI Disk Array shall be a Bosch/NetApp E2600 series that provides a scale-out network storage solution.

      - The iSCSI Disk Array shall offer redundant hot-swap power and cooling, dual 10 Gigabit iSCSI host ports, battery-backed cache, and protection from double-disk failure and single bit errors during RAID rebuilds with RAID-6.

      - The iSCSI Disk Array shall be a full-featured RAID protected disk array which provides advanced RAID-5 and RAID-6 protection.

      - The iSCSI Disk Array shall be based on one (1) 2U controller unit with 12 internal 3.5 in., 7.2 K, Serial Attached SCSI (SAS), 2 TB HDDs (Nearline SAS).

      - The iSCSI Disk Array shall offer two configurations:

        - Standard Simplex Controller

        - Dual Simplex Controller

      - [The The iSCSI Disk Array Standard Simplex Controller configuration shall support connection of up to seven (7) disk shelf expansion units (12 x 2 TB HDD per expansion shelf.)]

      - [The iSCSI Disk Array Dual Simplex Controller configuration shall support connection of up to three (3) high-density shelf expansion units (60 x 3 TB HDD per expansion shelf.)]

      - The iSCSI Disk Array shall offer a 4U expansion unit with 60 x 3.5 in., 7.2 K, SAS, 3 TB HDDs.

      - The iSCSI Disk Array shall offer two (2) 10 Gigabit Ethernet ports for high speed iSCSI connectivity.

    - Monitoring

      - The iSCSI Disk Array shall offer full SNMP support with MIB-II and NetApp custom MIBs supported.

      - The iSCSI Disk Array shall generate e-mail alert notifications in case of component failures.

      - The iSCSI Disk Array shall offer an Event Monitor to send alert notifications to a central Major Event Log.

    - Advanced System Features

      - The iSCSI Disk Array shall provide automatic drive failover, detection, and rebuild using global hot spare drives.

      - The iSCSI Disk Array shall provide mirrored data cache with battery backup and destage to flash

    - Reliability and Availability

      - The iSCSI Disk Array shall offer redundant hot-swappable power and fans.

      - The iSCSI Disk Array shall offer protection from double-disk failures and single bit errors during RAID rebuilds with high-performance RAID 6.

      - The iSCSI Disk Array shall offer automatic drive failover and detection and rebuild using global hot spare drives.

      - The iSCSI Disk Array shall offer mirrored data cache with battery backup and destage to flash.

      - The iSCSI Disk Array shall offer SANtricity Proactive Drive Health monitoring.

    - Management Features

      - The iSCSI Disk Array shall offer two (2) 1 Gigabit Ethernet ports for management access.

      - The iSCSI Disk Array shall offer a Command Line Interface (CLI) which is remotely accessible by Telnet, SSH, or RSH.

      - The iSCSI Disk Array shall provide a maximum bandwidth of 800 Mbps with a maximum of 400 concurrent iSCSI connections.

      - The iSCSI Disk Array shall offer central management through an intuitive graphical user interface.

      - The iSCSI Disk Array shall offer SANtricity Proactive Drive Health to identify problem drives.

      - SANtricity shall be capable of installation on a 32-bit or 64-bit Windows client.

      - SANtricity shall support on-the-fly expansion, reconfiguration, and maintenance without interrupting storage system I/O.

    - Net Capacity

      - RAID-5

        - Controller Unit

          - 8 x 2 TB: 13034 GB

          - 12 x 2 TB: 20482 GB

        - Expansion Unit

          - 12 x 2 TB: 20482 GB

          - 60 x 3 TB: 153670

      - RAID-6

        - Controller Unit

          - 8 x 2 TB: 11172 GB

          - 12 x 2 TB: 18620 GB

        - Expansion Unit

          - 12 x 2 TB: 18620 GB

          - 60 x 3 TB: 139700

    - Base Unit – DSA-N2E6X2-08AT

      - Electrical Requirements – Controller Unit

        - Nominal Voltage

          - Low Range: 100 VAC

          - High Range: 240 VAC

        - Frequency

          - Low Range: 50 to 60 Hz

          - High Range: 50 to 60 Hz

        - Idle Current

          - Low Range: 3.97 A

          - High Range: 1.63 A

        - Maximum Operating Current

          - Low Range: 4.25 A

          - High Range: 1.68 A

        - Sequential drive group spin up

          - Low Range: 4.27 A

          - High Range: 1.76 A

        - Simultaneous drive spin up

          - Low Range: 6.13 A

          - High Range: 2.17 A

        - System rating plate label

          - Low Range: 7.0 A

          - High Range: 2.9 A

      - Power Ratings

        - KVA: 0.400

        - AC Watts: 399

        - BTU/H: 1366

      - System Hardware Specifications

        - Chassis: CE2600 controller-drive tray conforms to the 483 mm(19.0 in) rack standard

        - Power supplies: Dual redundant, hot pluggable

        - Maximum disk drives: 8 x 2 TB, 3.5-in., Nearline SAS

        - Maximum net capacity: ~13000 GB (RAID-5 configured)

        - DDR2 memory (system RAM): 2 GB

        - Integrated I/O: 2 x 100 Mbps management port; 2 x 1/10 Gigabit Ethernet, copper

        - Remote management (via LAN): Yes (plus serial console on dedicated port)

      - Mechanical

        - Dimensions (H x W x D)

          - Controller-drive tray: 86.4 x 482.6 x 552.5 mm (3.4 x 19 x 21.75 in)

          - Shipping carton: 241.3 x 584.2 x 685.8 mm (9.5 x 23 x 27 in)

        - Weight

          - Maximum weight: 23 kg (55.72 lb)

          - Maximum shipping weight: 27.75 kg (61.2 lb)

      - Space Requirements

        - Front-cooling: 760 mm (30 in) clearance in front of the controller

        - Rear-cooling: 610 mm (24 in) clearance behind the controller

        - Front of chassis-maintenance: 760 mm (30 in) clearance in front of the cabinet

        - Rear of chassis-maintenance: 610 mm (24 in) clearance behind the cabinet

      - Environmental

        - Temperature

          - Operating range (both cabinet and subsystem): +10 °C to +35 °C 
(+50 °F to +104 °F)

          - Maximum rate of change: 10 °C (50 °F) per hour

          - Storage range: -10 °C to +50 °C (+14 °F to +122 °F)

        - Relative humidity (non-condensing)

          - Operating range (both cabinet and subsystem): 20 to 80%

          - Storage range: 10 to 90%

        - Acoustic noise

          - Sound power (standby operation): 6.5 bels max

          - Sound pressure (normal operation): 65 dBA max

    - Base Unit – DSA-N2E6X2-12AT / DSA-N2C6X2-12AT

      - Electrical Requirements – Controller Unit

        - Nominal Voltage

          - Low Range: 100 VAC

          - High Range: 240 VAC

        - Frequency

          - Low Range: 50 to 60 Hz

          - High Range: 50 to 60 Hz

        - Idle Current

          - Low Range: 3.97 A

          - High Range: 1.63 A

        - Maximum Operating Current

          - Low Range: 4.25 A

          - High Range: 1.68 A

        - Sequential drive group spin up

          - Low Range: 4.27 A

          - High Range: 1.76 A

        - Simultaneous drive spin up

          - Low Range: 6.13 A

          - High Range: 2.17 A

        - System rating plate label

          - Low Range: 7.0 A

          - High Range: 2.9 A

      - Power Ratings

        - KVA: 0.400

        - AC Watts: 399

        - BTU/H: 1366

      - System Hardware Specifications

        - Chassis: CE2600 controller-drive tray conforms to the 483 mm(19.0 in) rack standard

        - Power supplies: Dual redundant, hot pluggable

        - Maximum disk drives: 12 x 2 TB, 3.5-in., Nearline SAS

        - Maximum net capacity: ~20500 GB (RAID-5 configured)

        - DDR2 memory (system RAM): 2 GB

        - Integrated I/O: 2 x 100 Mbps management port; 2 x 1/10 Gigabit Ethernet, copper

        - Remote management (via LAN): Yes (plus serial console on dedicated port)

      - Mechanical

        - Dimensions (H x W x D)

          - Controller-drive tray: 86.4 x 482.6 x 552.5 mm (3.4 x 19 x 21.75 in)

          - Shipping carton: 241.3 x 584.2 x 685.8 mm (9.5 x 23 x 27 in)

        - Weight

          - Maximum weight: 24.9 kg (54.9 lb) / 27 kg (59.52 lb)

          - Maximum shipping weight: 29.65 kg (65.37 lb) / 31.75 kg (70.0 lb)

      - Space Requirements

        - Front-cooling: 760 mm (30 in) clearance in front of the controller

        - Rear-cooling: 610 mm (24 in) clearance behind the controller

        - Front of chassis-maintenance: 760 mm (30 in) clearance in front of the cabinet

        - Rear of chassis-maintenance: 610 mm (24 in) clearance behind the cabinet

      - Environmental

        - Temperature

          - Operating range (both cabinet and subsystem): +10 °C to +35 °C 
(+50 °F to +104 °F)

          - Maximum rate of change: 10 °C (50 °F) per hour

          - Storage range: -10 °C to +50 °C (+14 °F to +122 °F)

        - Relative humidity (non-condensing)

          - Operating range (both cabinet and subsystem): 20 to 80%

          - Storage range: 10 to 90%

        - Acoustic noise

          - Sound power (standby operation): 6.5 bels max

          - Sound pressure (normal operation): 65 dBA max

    - Expansion Unit – DSX-N1D6X2-12AT

      - Electrical Requirements – Controller Unit

        - Nominal Voltage

          - Low Range: 100 VAC

          - High Range: 240 VAC

        - Frequency

          - Low Range: 50 to 60 Hz

          - High Range: 50 to 60 Hz

        - Idle Current

          - Low Range: 2.96 A

          - High Range: 1.23 A

        - Maximum Operating Current

          - Low Range: 3.03 A

          - High Range: 1.26 A

        - Sequential drive group spin up

          - Low Range: 4.23 A

          - High Range: 1.76 A

        - Simultaneous drive spin up

          - Low Range: 4.43 A

          - High Range: 1.83 A

        - System rating plate label

          - Low Range: 7.0 A

          - High Range: 2.9 A

      - Power Ratings

        - KVA: 0.276

        - AC Watts: 276

        - BTU/H: 954

      - System Hardware Specifications

        - Chassis: DE1600 controller-drive tray conforms to the 483 mm(19.0 in) rack standard

        - Power supplies: Dual redundant, hot pluggable

        - Maximum disk drives: 12 x 2 TB, 3.5-in., Nearline SAS

        - Maximum net capacity: ~20500 GB (RAID-5 configured)

        - Integrated I/O: 6 Gb SAS

      - Mechanical

        - Dimensions (H x W x D)

          - Controller-drive tray: 86.4 x 482.6 x 552.5 mm (3.4 x 19 x 21.75 in)

          - Shipping carton: 241.3 x 584.2 x 685.8 mm (9.5 x 23 x 27 in)

        - Weight

          - Maximum weight: 27 kg (59.52 lb)

          - Maximum shipping weight: 31.75 kg (70.0 lb)

      - Space Requirements

        - Front-cooling: 760 mm (30 in) clearance in front of the controller

        - Rear-cooling: 610 mm (24 in) clearance behind the controller

        - Front of chassis-maintenance: 760 mm (30 in) clearance in front of the cabinet

        - Rear of chassis-maintenance: 610 mm (24 in) clearance behind the cabinet

      - Environmental

        - Temperature

          - Operating range (both cabinet and subsystem): +10 °C to +40 °C 
(+50 °F to +104 °F)

          - Maximum rate of change: 10 °C (50 °F) per hour

          - Storage range: -10 °C to +50 °C (+14 °F to +122 °F)

        - Relative humidity (non-condensing)

          - Operating range (both cabinet and subsystem): 20 to 80%

          - Storage range: 10 to 90%

        - Acoustic noise

          - Sound power (standby operation): 6.5 bels max

          - Sound pressure (normal operation): 6.8 bels max

    - Expansion Unit – DSX-N6D6X3-60AT

      - Electrical Requirements – Controller Unit

        - Nominal Voltage

          - Low Range: 100 VAC

          - High Range: 240 VAC

        - Frequency

          - Low Range: 50 to 60 Hz

          - High Range: 50 to 60 Hz

        - Idle Current

          - Low Range: 6.0 A

          - High Range: 6.0 A

        - Maximum Operating Current

          - Low Range: 7.56 A

          - High Range: 7.56 A

      - Power Ratings

        - KVA: 1.268

        - AC Watts: 1222

        - BTU/H: 4180

      - System Hardware Specifications

        - Chassis: DE6600 controller-drive tray conforms to the 483 mm(19.0 in) rack standard

        - Power supplies: Dual redundant, hot pluggable

        - Maximum disk drives: 60 x 3 TB, 3.5-in., Nearline SAS

        - Maximum net capacity: ~153670 GB (RAID-5 configured)

        - Integrated I/O: 6 Gb SAS

      - Mechanical

        - Dimensions (H x W x D)

          - Controller-drive tray: 177.8 x 482.6 x 825.5 mm (7.0 x 19 x 32.5 in)

          - Shipping carton: 482.6 x 609.6 x 1009.7 mm (19 x 24 x 39.75 in)

        - Weight

          - Maximum weight: 105.2 kg (232 lb)

          - Maximum shipping weight: 193.2 kg (426 lb)

      - Space Requirements

        - Front-cooling: 760 mm (30 in) clearance in front of the controller

        - Rear-cooling: 610 mm (24 in) clearance behind the controller

        - Front of chassis-maintenance: 760 mm (30 in) clearance in front of the cabinet

        - Rear of chassis-maintenance: 610 mm (24 in) clearance behind the cabinet

      - Environmental

        - Temperature

          - Operating range (both cabinet and subsystem): 0 °C to +35 °C
(+32 °F to +95 °F)

          - Maximum rate of change: 10 °C (50 °F) per hour

          - Storage range: -10 °C to +50 °C (+14 °F to +122 °F)

        - Relative humidity (non-condensing)

          - Operating range: 20 to 80%

          - Storage range: 10 to 90%

        - Acoustic noise

          - Sound power (standby operation): 6.5 bels max

          - Sound pressure (normal operation): 6.8 bels max

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to disk arrays for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of array settings to comply with specific customer’s need.

  - DEMONSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION