- Master -

FINAL  FORKLIFT  TEST

1.	Your forklift can be driven by:

a.  Any employee.

b.  An employee who has training from another facility.

c.  Only operators specially trained by their company.

2.	When changing directions: (forward to reverse, or reverse to forward)

a.  Always stop.

b.  Slow down to about 1 mph.

c.  Don’t worry about speed; a forklift is built to take shock.

3.	Never exceed the rated load capacity of your truck:

a.  Unless you add additional counterweight.

b.  Unless your lift is under 84 inches high.

c.  No, never.

4.	When taking a load down a ramp:

a.  Drive backwards.

b.  Drive forwards.

c.  Reducing speed by zigzagging.

5.	If you have to park on a ramp:

a.  Leave the machine in gear.

b.  Set the brakes and block the wheels.

c.  Set the load down, and rest the machine against it.

6.	On a forklift, your horn:

a.  Makes a good device to catch the foreman's attention.

b.  Should be sounded at intersections.

c.  Should be sounded when you're stacking pallets.

7.	When loading a truck trailer, you should:

a.  Check the trailer tires to make sure they are blocked.

b.  Check to make sure that the tractor is attached.

c.  Load the left side of the trailer first.

8.	When approaching a blind corner, you should:

a.  Stop, and then proceed.

b.  Slow down, sound the horn and proceed slowly.

c.  Continue at same speed while sounding the horn.

9.	The condition of a pallet:

a.  Affects load stability.

b.  Affects load center.

c.  Neither of the above.

10.	The speed at which a forklift should be operated is:

a.  Determined by the operator.

b.  Determined by the load, driving conditions, and plant rules.

c.  Determined by signs posted along aisles and passageways.

11.	When operating a fully loaded sit down riding type forklift on a ramp, you should:

a.  Drive up the ramp with the load in front.

b.  Drive up the ramp with the load trailing.

c.  It doesn't make a difference.

12.	When parking a lift truck:

a.  The forks should be above eye-level to prevent an accident.

b.  2" above the floor.

c.  Resting on the floor with the tips pointing down.

13.	Safety of a forklift is the responsibility of:

a. Operators

b. Management

c. Manufacturer

- All of the above

14.	After picking up a palletized load, it should be tilted:

a.  As far back as possible.

b.  Only far enough back to stabilize the load.

c.  Not at all.

15.	Forklift operator training is:

a.  Required by law.

b.  Is only necessary for new operators.

c.  Is valuable even if it only prevents one accident from occurring.

d.  Both "a" and "c".

TRUE  OR  FALSE

16.	Your tires should be checked every time you use the truck. - T

17.	A lift truck steers with its rear wheels and its rear end swings on turns. - T

18.	It is okay to smoke when refueling as long as you are in a restricted area. - F

19.	Start out slowly with every load. - T

20.	Loads when lifted high reduce a truck's stability. - T

21.	As long as you sit beside him, it is okay to let an unauthorized person operate your forklift. - F

22.	Never let bystanders stand beneath loads when you're stacking. - T

23.	A careful driver may hang his leg outside a machine when it is protected by a wide load. - F

24.	A good operator drives slowly on slippery and wet floors. - T

25.	One of the greatest causes of accidents is forgetting to watch overhead obstructions when lifting. - T

26.	An overhead guard is intended to offer protection from the impact of falling capacity loads. - F

27.	The load forks can be used to lift people, if they are careful. - F

28.	The center of the load should be situated on the tips of the load forks. - F

29.	The operator does not have to inspect his truck after a mechanic has made repairs. - F

30.	Forklifts should not be parked in front of stairways and fire equipment. - T