# Pre-Trip Inspection Procedure

Stations 1, 3, 6, 10 and 12   Wheels and Brakes

- Inspect wheels for cracks, bent rims and broken studs, clamps and lugs. Check the inflation and valve stems, look for cuts, bulges, tread wear and signs of misalignment.

- Inspect wheel bearing and hub for leaking.

- Dual wheels should be evenly matched with the same type of tires.

- Check the condition of the brake drums, hoses, air chamber mounting and slack adjusters.

Station 2   Front Cab

- Check steering wheel system for damaged parts, inspect windshield and wiper blades, check lights, signal indicators and reflectors.

Stations 4 and 14   Saddle Tanks

- Fuel tanks and caps should be secure and not leaking.

- Check for leaks from the rear of the engine, transmission, drive shaft and exhaust system.

- Air lines and electrical wiring should be secure.

- Check frame and cross members for damage

- Battery and battery box should be secure: check battery fluid and cell caps.

Station 5   Trailer Front

- Glad hands should be properly mounted, undamaged and not leaking.

- Check the seating and safety catch on electrical plugs and secure air and electrical lines against tangling, snagging and chafing, with sufficient slack for turns.

Station 7   Rear of Tractor

- Cross members should not be bent or cracked.

- Lights and reflectors should be clean and operating.

- Secure air lines to frame.

Station 8   Fifth Wheel

- The fifth wheel should be securely mounted to the frame with no missing or damaged and no visible space between the upper and lower fifth wheel.

- Locking jaws should secure the shank, not the head of the kingpin, and the release lever should be properly seated and the safety lock engaged.

Stations 9 and 13   Trailer Sides

- Landing gear and dollies should be raised.

- Check for air or hydraulic leaks.

- Check the spare tire rack for damage and secure the wheel after checking its inflation level.

- Lights should be clean and operating.

- All placards should match.

Station 11   Trailer Rear

- Cargo and tailboard should be secured and canvas or tarp lashed down.

- Latch and lock rear doors and check the underguard.